RFDK34-18G-72F Double knitting specification

Cylinder Dia：34”   Feeder No：72F    gauge：18G   needle no：1920T

| S/N | Type | Part name | unit | quantity | Spare quantity | brand | remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 122.48 | needle | Pcs | 960 | 50 | groz | 1/2 |
| 2 | 74.50 | needle | Pcs | 960 | 50 | groz | 1/2 |
| 3 | H01 | Bellow knit cam | Pcs | 72*2 | 1 | RIFA |  |
|  | H02 | Bellow Tuck cam | Pcs | 72*0.5 | 1 | RIFA |  |
|  | H03 | Bellow Miss cam | Pcs | 72*1.5 | 1 | RIFA |  |
|  | F01/F04 | Up knit cam | Pcs | 72*1 | 1 | RIFA |  |
|  | F02/F05 | Up Tuck cam | Pcs | 72*0.25 | 1 | RIFA |  |
|  | F03/F06 | Up miss cam | Pcs | 72*0.75 | 1 | RIFA |  |
| 4 | ceramics | Vertical yarn guide | Pcs | 72 | 1 | RIFA |  |
| 5 |  | Wire bearing | Set | 1 |  | England |  |
| 6 | Middle tune | Knit part | Set | 1 |  | RIFA |  |
| 7 | 34*18G*1920T | Plug type cylinder | Set | 1 |  | RIFA |  |
| 8 | key type | control panel | Set | 1 |  | RIFA |  |
| 9 | 5.5KW/4P | inverter | Set | 1 |  | Delta |  |
| 10 | 5.5KW/4P | motor | Set | 1 |  | Hongqi |  |
| 11 | Double Reversible with length 1.3M | Cloth rolling machine | Set | 1 |  | RIFA |  |
| 12 | 093 | Auto oiler | Set | 1 |  | Hongyu |  |
| 13 | 213上段 | Upper dust clear | Set | 1 |  | Hongyu |  |
| 14 | APF20-T29 | Yarn feeder | Set | 72 | 1 | Saimeige |  |
| 15 | SER-T5 | Lycra feeder | Set | 18 |  | Saimeige |  |
| 16 | 9.6m | Belt(gear type) | Pcs | 1 |  | Men |  |
| 17 | 10.2m | Belt(gear type) | Pcs | 1 |  | Men |  |
| 18 | 12m | Belt(gear type) | Pcs | 1 |  | Men |  |
| 19 | Triangle Tube Creel（8 hole） | Side creel | Set | 1 |  | RIFA |  |
| 20 | Three circle | Top feeder | Set | 1 |  | RIFA |  |
| 21 | Single layer | Speed plate (250) belt adjustment frame | Pcs | 1 |  | Saimeige |  |
| 22 | Double layer | Speed plate (210) belt adjustment frame | Pcs | 2 |  | Saimeige |  |
| 23 |  | Tools | Set |  | 1 set for each 5 machines |  |  |
| Note: The configuration table is a single configuration list |  |  |  |  |  |  |  |  |