RFSK34-18G-102F Single knitting specification

Cylinder Dia：34”   Feeder No：102F    gauge：18G   needle no：1920T

| S/N | Type | Part name | unit | quantity | Spare quantity | brand | remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 141.52 | needle | Pcs | 960 | 50 | groz | 1/2 |
| 2 | 0.3/0.15 | sinker | Pcs | 1920 | 100 | kern |  |
| 3 | H01 | Bellow knit cam | Pcs | 204 | 1 | RIFARIFA |  |
|  | H02 | Bellow tuck cam | Pcs | 102 | 1 | RIFA |  |
|  | H03 | Bellow miss cam | Pcs | 102 | 1 | RIFA |  |
|  | S01 | Upper cam | Pcs | 102 | 0 | RIFA |  |
| 4 | Pulley | Vertical yarn guide | Pcs | 102 | 1 | RIFA | Sanda |
| 5 |  | Wire bearing | Set | 1 |  | England |  |
| 6 | Middle tune | Knit part | Set | 1 |  | RIFA |  |
| 7 | 34*18G*1920T | Plug type cylinder | Set | 1 |  | RIFA |  |
| 8 | key type | Control panel | Set | 1 |  | RIFA |  |
| 9 | 5.5KW/4P | inverter | Set | 1 |  | delta |  |
| 10 | 5.5KW/4P | motor | Set | 1 |  | Hongqi |  |
| 11 | Single Reversible with length 1.3M | 摆布机总成 | Set | 1 |  | RIFA |  |
| 12 | 093 | Auto oiler | Set | 1 |  | Hongyu |  |
| 13 | 213 | Upper dust clear | Set | 1 |  | Hongyu |  |
| 14 | 213 | Middle dust clear | Set | 1 |  | Hongyu |  |
| 15 | APF20-T29 | Yarn feeder | Set | 102 | 1 | Saimeige |  |
| 16 | SER-T5 | Lycra feeder | Set | 26 |  | Saimeige |  |
| 17 | 9.6m | Belt(gear type) | Pcs | 1 |  | Men |  |
| 18 | 9.8m | Belt(gear type) | Pcs | 1 |  | Men |  |
| 19 | 11.6m | Belt(gear type)） | Pcs | 1 |  | Men |  |
| 20 | Triangle Tube Creel（8 hole） | Side creel | Set | 1 |  | RIFA |  |
| 21 | Three circle | Top feeder | Set | 1 |  | RIFA |  |
| 22 | Single layer | Speed plate (250) belt adjustment frame | Pcs | 1 |  |  |  |
| 23 | Double layer | Speed plate (210) belt adjustment frame | Pcs | 2 |  |  |  |
| 24 |  | Tools | Set | 1 set for each 5 machines |  |  |  |
| 25 | Other |  |  |  |  |  |  |
| Note: The configuration table is a single unit configuration list. |  |  |  |  |  |  |  |