MACHINE SPECIFICATIONS

| STT | INFORMATION | TECHNICAL REQUIREMENTS | HENGTAI CONFIRM |
| --- | --- | --- | --- |
| 1 | Machine MODEL | HT-HF450L |  |
| 2 | PRODUCT | TERRY TOWEL |  |
| 3 | Towel width | 200-450mm |  |
| 4 | Finished towel lenght | 300-1280mm |  |
| 5 | Towel weight | 190-600g/m2 |  |
| 5 | Hem width | 10-25mm |  |
| 6 | Sewing stitch distance | 2.5-5mm/stitch |  |
| 7 | Back stitch leght | Free to set(0.1-2.5mm/stitch) |  |
| 8 | Fret lenght | 74-500mm |  |
| 9 | Sewing | Lockstitch |  |
| 10 | Number of labeling machines | 01 machine |  |
| 11 | Label location | Left/right as requested by the customer and attached at the top of the towel |  |
| 12 | Label supply type | Roll style label |  |
| 13 | Label size | length of label 30mm width of label 140mm |  |
| 14 | Label material: | textile, paper, sateen, nylon… |  |
| 15 | Port to connect to the computer | RJ45 |  |
| 16 | Save machine operating data |  |  |
| 17 | Voltage/power | 1phase 220v/50hz 3.6kw |  |
| 18 | Maximum air consumption flow rate | 5kg/cm3 |  |
| 19 | Net weight | 2200kg |  |

Zhejiang Hengtai intelligent device co.ltd