ANNEX 1

- Automatic edge sewing machine

| No | Category | Hengtai |
| --- | --- | --- |
|  | Model | HT-1800 |
|  | Year of manufacture | 2024 |
|  | Quantity | 01 |
| 1 | Product | Product: Terry towel (large, thick borders) |
| 2 | Thread | 20/2, 20/3, 40/2, 50/3 Cotton or PE |
| 3 | Weight of towel | 190g-855g/m2 |
| 4 | Width of towel | From 25cm to 180cm |
| 5 | Length of towel | from 20cm to 200cm |
| 6 | Stitch | Chainstitch |
| 7 | Seam | 2 parallel threads, 2 parallel chainstitch |
| 8 | Seam pitch | 1.8 - 6.0 stitch/1cm |
| 9 | Width of scarf edge after sewing | Standard 7mm scarf folding set, with scarf trimming kit |
| 10 | Label dispenser | 1 set standard 1 set options |
| 11 | Label quantity | 2 labels/scarf edge |
| 12 | Labeling specifications | Two labelers are located on the left side, the hanging labeler is in front, the standard labeler is in the back |
| 13 | Label granting structure | Roll |
| 14 | Label material | Woven, sateen, nyloncros, paper |
| 15 | Label type | 1. Loop label width: 10-60mm length: 30-170mm |
|  |  | 2. Flag label width: 10-60mm length: 30-170mm |
|  |  | Hanger label: width: 10-20mm length: 70-110mm |
| 16 | Machine speed | 12-14m/minute |
| 17 | Locomotive brand | PEGASUS |
| 18 | Voltage | - 3 phase 380v 50Hz |
|  |  | - HMI control screen |
| 19 | List of spare parts |  |
|  | Needle set for sewing machine head | 2 pcs |
|  | The uvula gives the border fold | 1 set |
|  | Upper left and right cutter set | 10 pcs |
|  | Lower left and right cutter set | 10 pcs |
|  | Magnetic sensor on piston | 2 pcs |
|  | Synchronous belt set for sewing machines | 2 rolls |
|  | Accessory set for hanging label dispenser | 1 set |
|  | Technical documents, operating instructions, electrical diagrams | 01 set |